Slowdos v2.3 - 2.3.98 (Full version)

------------------------------------

Slowdos v2.x  (hereafter referred to as Slowdos), is an

extension to +3 BASIC which allows MSDOS discs to be read

and written from BASIC

Although there have already been a couple of programs which can

be run on the +3 which allow MSDOS discs to be used, they are

limited in scope: The original Slowdos only copied files, and

MSODBALL (by John Elliot) runs only from CP/M and uses an

obscure format which could be read by both MSDOS and CP/M.

Enter in here Slowdos, the program which allows all manner of

functions to be performed on MSDOS discs from BASIC.

System Requirements

-------------------

Slowdos will run on any +2a or +3 (hopefully regardless of

issue), however for practical usage a +3 with an external

disc-drive is recommended.

The external drive should be double-sided and capable of

reading 80 tracks ie a DSDD disc-drive. Slowdos has been

developed with 3.5" drives in mind, though there is no reason

why it shouldn't work with a DSDD 5.25" drive.

It is possible that Slowdos will work with single density

drives, although they to all intents and purposes obsolete, but

I make no guarantees as to data integrity.

Distribution

------------

Slowdos is the culmination of over 3 years of work and is thus

a shareware program. I believe that it is excellent value for

money at `6 bearing in mind how much it extends the flexibility

of the +3. The full version may not be distributed in any form.

Anyone found distributing the full version should expect a

legal experience. There exists a cut down version (v2.0) which

may be distributed for evaluation purposes.

Disclaimer

----------

Slowdos is supplied as is, and therefore I (Dominic Morris)

will not be held responsible for any damage to you, your

computer, your data. In addition, I explicitly disclaim

anything that can be disclaimed.

The new commands

----------------

A definitive list of commands now follows, it is not intented

to be a tutorial as to how to use them, it is presumed that the

concepts behind them is already known from use of the

equivalent +3 commands.

Key to symbols used:

a(n) = 16 bit number

dd   = drive no 1,2,*

f    = filename

u1   = source filename

u2   = destination filename

ss   = stream number

All parameters in parenthesis [ ] are optional.

Essential disc filing commands

------------------------------

CAT [#ss,]dd"f"

Catalogues a disc, to stream ss (default is the screen), the

file name may be wild.  The displayed format is identical to

that of a normal +3 drive. NB Hidden (system) files are

automatically shown.

LOAD Pdd"f"

Load a file from the disc, all the usual BASIC extensions may

be used ie CODE, DATA, SCREEN$. In addition the extension S can

be used to load a 48k snapshot.

The snapshot format is the .SNA format, supported by all

emulators and is the de facto standard for 48k snapshots.

SAVE Pdd"f"[!]

Save a file to disc, as with LOAD all the usual BASIC

extensions may be used. If ! is specified before any of the

BASIC extensions then the file is saved without a PLUS3DOS

compatible header

MERGE Pdd"f"

Merge a BASIC file on disc with the one in memory

ERASE Pdd"f"

Erase the file f from the disc

FORMAT Pdd[!][;"f"]

Format the current disc in DSDD MSDOS format. Confirmation is

asked prior to formatting. If ! is specified then the disc will

not be formatted, but merely initialised. The filename f is

the disc name, this is not used by Slowdos, but is provided for

ease of use of Slowdos formatted discs on other machines.

Advanced disc commands

----------------------

ERASE Pdd"u1" TO "u2"

Rename the file u1 giving it the name u2

MOVE Pdd"f" TO #ss

Copy the file f to the stream ss, note that file filtering

takes place, all character codes below 32 are replaced

with a space.

COPY Pdd"u1" TO "u2"

Copies the file named u1 on the MSDOS disc to the file named u2

on the default +3 drive. u2 may be a drive in which case u1 can

be wild, keeping the original filenames. Note that all the

files saved on the +3 disc will be saved with PLUS3DOS headers,

and if appropriate the headerdata filled to create files

loadable by +3 BASIC. The default for unrecognised files of

length shorter than 65535 bytes is to be a code file with a

load address of 0. 48k snapshots are saved as filetype 4.

COPY "u1" TO Pdd[!]["u2"]

Copy the file u1 from the default +3 drive to the MSDOS disc.

If u1 is wild then omit u2. If the ! parameter is supplied then

the files saved on the MSDOS disc will be headerless.

LOAD @dd,tt,ss,aa

SAVE @dd,tt,ss,aa

These commands are used for reading/ writing disc sectors from

memory.  tt is the track, 0 - 79 (side 1), 80-159 (side 2). ss

is the sector number (1-9), and aa is the memory address in

memory configuration 5,2,0 (normal configuration)

MOVE TO Pdd"f"

Change directory, equivalent to the CD command on larger

computers. Inside each subdirectory there are always two

files. A "." file which refers to the current directory

and a ".." which points to the parent directory.

A full path may be specifed by stringing directories together

thus:

MOVE TO pdd"dir1/dir2/dir3...."

Note the use of the / not \ - this is in line with all real

operating systems, UNIX, VAX, AmigaDOS etc

To move back to the root directory leave the pathname out or

make it null.

Once inside a subdirectory, usage of Slowdos is not affected,

it is still possible to load/save/copy/rename etc files.

Compatibility Commands

----------------------

VERIFY Ddd"f"

This commands are inoperative, they pass syntax to avoid

problems with programs that do use them.

POKE @a1,a2

Does a 16 bit poke for the entire memory configuration (ROM3,

pages 5,2,7). This command is mainly supplied for small patches

to Slowdos (should they be needed), however it does have other

uses - try POKE 60433,col and POKE 60431,col to change the

editing colours for the top and bottom of the screen.

Accessing Spectrum Emulator Files

---------------------------------

Slowdos accesses a number of file formats used by emulators on

other machines. The most common of these is the snapshot file,

the usage of this was detailed above, but to recap:

LOAD Pdd"f"S  will load a 48K .SNA file and lock the machine in

48k mode.

LOAD Pdd"f"S! will load a 48k .SNA but will not lock the +3 in

48k mode. If you manage to break out of the

program then the chances are that you will be in 48k mode, but

you can return to 128k with Slowdos running via a PRINT USR

23354. Please note with this that crashes/spurious effects may

sometimes result whilst using the snapshot due to the fact that

data in the printer buffer isn't loaded.

Slowdos also supports the .TAP file format. A TAP file is in

essence an image of a cassette tape, and is often used to

preserve the loading screen of a game, it is also used by

Alchemist Software to distribute Alchnews for use on emulators.

Slowdos treats TAP files as (limited) sub-directories, limited

in the respect that they are not random access nor can a

CATalogue be performed. However it is possible to copy files

out of a TAP file to +3 disc.

To enter a TAP file use the command

MOVE TO Tdd"f"[IN]

Please note the use of T and not P (as for subdirectories). The

filename f must have the extension .TAP.

Once inside a TAP file the command LOAD P*"" may be issued to

load the first BASIC file found within it. As you may have

deduced it is necessary to prefix the filename with a P* - this

is so as to fail +3 syntax and to pass the command to Slowdos.

As a result it should be obvious that any program within the

TAP which loads files from BASIC will have to slightly modified

- namely the P* inserted before the filename. I believe that

this is a minor inconvenience when compared to the flexibility

that being able to access a TAP file provides.

Following logically, any program which loads headerless files

will not work, however it is possible to copy headerless files

to +3 disc, and then modify the program so that it does work

on a +3 - in essence converting the program back to its native

computer. The syntax of the COPY command is exactly the same

as for dealing with normal directories.

MOVE TO Tdd"f"OUT

This is a very powerful command which allows files on the +3 to

export files for use with emulators.

The file format used is (not unsuprisingly) the .TAP format, to

use it you can either copy files to it, or save files into it.

To close the .TAP file (when all the files are enclosed) do a

MOVE TO Tdd. The file will automatically be closed, and the

directory entry written. Please note that if you forget to do

this, then the .TAP willnot be accessable, but space on the

disc will have been allocated.

As a little note to avoid any potential confusion, if you are

reading a .TAP file, any writing will take place in the current

directory, and if you are writing a .TAP file any reading will

access the current directory (on the MSDOS disc).

Unfortunately due to memory constraints it isn't possible to

copy files into a .TAP file from an MSDOS disc (and vice

versa).

Usage of drives

---------------

Throughout the list of commands reference has been made to

drive numbers, however, since the +3 only supports one external

drive, there is only one MSDOS drive,and thus p1 and p2 access

the same physical drive.

Although the external drive is used by Slowdos, it is still

possible to perform +3 DOS functions on the B: drive, though

obviously it isn't possible to use both drives at the same time

eg copy from B: drive to MSDOS disc.

To this end, and in order to prevent corruption of +3 discs,

only discs which match a template MSDOS disc format will be

used by Slowdos, which rejects for use as MSDOS disc any

BFORMAT +3 discs. (Obviously it is possible to reformat these

discs for use by Slowdos).

Error messages

--------------

All of the routines are fully error trapped and will return to

BASIC with a harmless error message should anything untoward

occur. In addition the RIC messages from +3 DOS have been

disposed of, any errors now result in an error message.

General notes

-------------

It is well known that the +3 has problems driving certain

printers. A set of pokes was found which solved the problem,

however these shouldn't be used when Slowdos is installed

(crashes may result if you do), instead use POKE @49286,16

which will enable these problem printers to work. - Please note

that this POKE has changed from earlier versions of Slowdos and

DiSCDOS.

Slowdos survives the 128k NEW command, and need only be

reloaded if a reset is performed.

Slowdos occupies the second screen (in page 7), and as such

nothing should be placed there lest the +3 crash in an

interesting manner! It also uses some additional space in page

7, this doesn't interfere with normal usage of the +3 though it

may cause some problems with other programs.

Known bugs/feechures

--------------------

Because of the +3 CAT syntax problem any CAT phrase involving a numeric variable will not be able to be entered unless the

variable(s) exists. All other comands are unaffected, so the

value of remedying it seems dubious.

History

-------

v1.0      Original version of program released back in 1994

??.8.94

v1.1      Bug release

??.12.94

v2.0      Crippled demo version of the new Slowdos

5.12.97

v2.1      Fully featured version of the new Slowdos

7.12.97

(intern)

v2.2      Added .TAP, subdirectory support. Added the ! option

16.12.97  to snapshot loading. Optimized the code a lot.

v2.3      Added writing to .TAP files, added in system calls

2.3.98    (see Appendix II), incredible amount of optimization

made - some 300 bytes saved! Sorted out erasing of

files created on Sparc stations, cleaned up a

potential problem in SAVEing of arrays.

Acknowledgements

----------------

Thank you to: (alphabetical order of course!)

Ian Collier

For giving me information on the +3 ROMs for the DiSCDOS

project, without that, nothing would have been possible.

Andy Davis

For support and encouragement.

John Garner

For support and encouragement.

And finally, thank you for your support in registering Slowdos.

Dominic Morris,

20 Greenhill Drive,

Malvern,

Worcs,

WR14 2BW

Email:    djm@jb.man.ac.uk    (Note change of address)

2.3.97

Appendix I - The snapshot file format

----------

The snapshot format supported by Slowdos is the common used

.SNA format derived from the Mirage Microdriver format. This is

the de facto standard for 48k snapshots and is supported by all

major emulators.

The format of the file is as follows:

Offset         Bytes     Description

0                1         i

1                2         hl'

3                2         de'

5                2         bc'

7                2         af'

9                2         hl

11               2         de

13               2         bc

15               2         iy

17               2         ix

19               1         Interrupt (bit 2=IFF2 1=EI/0=DI)

20               1         r

21               2         af

23               2         sp

25               1         Intmode (0=im0/1=im2/2=im2)

26               1         Border colour

27               49152     Ram dump (16384...65535)

Appendix II - System calls

-----------

As of v2.3, Slowdos gives you, the programmer, the ability to

access MSDOS discs in a simple way, without having to write

your own routines. Via these system calls you can do whatever

your imagination allows - for example you could read different

emulator file formats or you could write a file manager! - It's

up to you!

All of the calls, are referenced by calling address 49155 in

page 7 with l holding the routine number, at the moment, the

following routines are accessible:

1, r_mslog

Entry parameters:   none

Exit parameters:    standard

Log an MSDOS disc in, read in the FAT, check format

2, r_getpar

Entry parameters:   none

Exit parameters:    de=address of header in page 7

Get the address of the file header in page 7, this is 7 byte

block ala +3 DOS as is arranged as follows:

|----------------|-----|-----|-----|-----|-----|-----|-----|

|   Byte         |  0  |  1  |  2  |  3  |  4  |  5  |  6  |

|----------------|-----|-----|-----|-----|-----|-----|-----|

|Program         |  0  |File length|8000h/LINE |prog offset|

|Numeric array   |  1  |File length| xxx |name | xxx | xxx |

|Character Array |  2  |File length| xxx |name | xxx | xxx |

|Code            |  3  |File length| load addy | xxx | xxx |

|Snapshot        |  4  |File length| xxx | xxx | xxx | xxx |

|Longfile        |  5  |   32 bit file length  | xxx | xxx |

|----------------|-----|-----------------|-----|-----|-----|

3, r_rdopen

Entry parameters:   ix=ufia

Exit parameters:    standard

Open a file on the MSDOS disc for reading. If the file has a

standard +3 header, then this header is recognized, and the

header block filled out accordingly. If no header is present

and the file is <65536 bytes, then a header is faked such that

it is a CODE file with load address 0.

4, r_rdbyte

Entry parameters:   none

Exit parameters:    a=byte

Read a byte from the currently open file.

5, r_rdblok

Entry parameters:   de=address to load to

bc=number of bytes to read

h=page to load to

Exit parameters:    none

Read a chunk of code from the currently open file to memory in

the page given by h

6, r_wropen

Entry parameters:   ix=ufia

Exit parameters:    none

Open a file for reading, if the filetype in the ufia is specified as type 5, then no +3DOS style header will be created.

7, r_wrbyte

Entry parameters:   a=byte to write

Exit parameters:    none

Write the byte in a to the currently open file

8, r_wrblok

Entry parameters:   de=address of block

bc=length of block

h=RAM page

Write a block of memory to disc

9, r_wrclos

Entry parameters:   none

Exit parameters:    none

Close the currently open file, write the directory entry and

FAT.

10, r_erase

Entry parameters:   ix=ufia

Exit parameters:    none

Erase the files specified by the ufia - may be wild

11, r_snpload

Entry parameters:   none

Exit parameters:    none

Load the currently open reading file as a snapshot file, and

execute accordingly.

12, r_mscat

Entry parameters:   ix=ufia

Exit parameters:    none

Catalogue a disc directory matching the specifications of the

ufia.

13, r_mscatmem

Entry parameters:   ix=ufia

de=address to place catalogue

Exit parameters:    none

Catalogue the disc to an address in memory - make sure you

have sufficient room for the catalogue!!! Initialise the memory

with 0 before hand!!!

The format of the data supplied by the disc is unsorted and

is as follows:

byte 0    - number of files in directory (0-255)

The filename information is as follows:

ds   11   - filename (without the ".")

ds   1    - directory flags

ds   2    - first cluster

ds   4    - file length (32 bit)

- As can be seen, each directory entry requires 18 bytes of

space - this can amount to a lot - the routine does not respect

RAM pages so if you're not careful you could overwrite the

Slowdos code.

14,  r_seldir

Entry parameters:   de=path name

bc=length of path name

Exit parameters:    none

Move to the directory described by that in de

Notes:

If successful all of the routines will return with carry set,

if unsuccessful, carry will not be set, and a will hold the

"rst 8" error code - apologies for this, but memory is very

much at a premium!

The ufia (User File Information Area) is 20 bytes long and is

ordered as follows:

+0   db   drive (0,1) (irrelevant)

+1   db   stream (for r_mscat)

+2   ds   filename (12 characters including ".")

+14  db   directory flags (irrelevant)

+15  db   filetype                 }

+16  dw   filelength               } +3 header info

+18  dw   start address/LINE/name  }

+18  dw   offset to BASIC          }

Appendix III - Bonus program, Snapconv

------------

Snapconv is small program for the +3 which decomposes snapshot

files into smaller files that can be reconstructed to create a

+3 BASIC loadable program.

To use it, simply copy a snapshot file from an MSDOS disc the

default +3 drive, for practical purposes this must be A:.

If Snapconv is then loaded and the snapshot named entered, the

snapshot will be decomposed into 4 separate files, with

different extensions, but the same filename as the snapshot,

these files are as follows:

Extension      CODE parameters

.0            0,27    The registers detailed in Appendix I

.1        16384,6912  Screendump

.2        23296,1280  Printer buffer/BASIC area

.3        24576,40960 Main code

It is then up to you to use the parts as you desire!!

Snapconv is (C) 5.12.97 D.J.Morris

EOF